Product Specification Document

Bright Power™ Floor Finish Stripper

Page 1 of 2

CLASSIFICATION:

- Ultra heavy-duty floor finish stripper

SCOPE:

- Bright Power™ Floor Stripper is the most versatile and powerful floor stripper designed to remove the heaviest floor finish build-ups. The ultra concentrate formula provides a broad dilution range (1:8 – 1:24) to tackle the toughest floor stripping needs. Low odor formula. Use on vinyl and vinyl asbestos floor tiles, ceramic tile, terrazzo and concrete floors. Before use on asphalt tile, test use-dilution in a hidden area to ensure that bleaching, bleeding or softening does not occur. DO NOT use on linoleum, rubber, marble or wood floors.

FEATURES AND BENEFITS:

| Feature | Benefit |
| --- | --- |
| - Low odor, butyl free | - No need to move occupants, shut down areas of your facility or interrupt busy work schedules |
| - Removes heaviest build up in fewer passes | - Time and labor savings versus conventional floor strippers |
| - Broad dilution range | - Versatile to meet all floor stripping needs |

USAGE:

FOR INDUSTRIAL USE ONLY

DIRECTIONS:

- Apply solution liberally with mop and allow solution to dwell for 10 minutes (allow 15 minutes for heavy build-up). Do not allow solution to dry – add more as needed.

- Scrub thoroughly with black pad or stripping brush.

- Pick up with wet vacuum, auto scrubber or mop.

- Rinse floor with clean water and allow floor to dry completely before applying Ecolab floor finish.

DILUTIONS:

- 1:8 – Heavy build-up (10+ coats) / highly burnished finishes

- 1:10 – 1:15 Medium build-up

- 1:16 – 1:24 Light build-up

PHYSICAL AND CHEMICAL PROPERTIES:

| - Compositional Information | Water | 7732-18-5 |
| --- | --- | --- |
|  | Benzyl alcohol | 100-51-6 |
|  | Monoethanolamine | 141-43-5 |
|  | Ethylene glycol monohexyl ether | 112-25-4 |
|  | Potassium oleate | 143-18-0 |
|  | Potassium hydroxide | 1310-58-3 |
| - Physical State | Liquid |  |
| - Color | Amber (Light) |  |
| - Odor | Amine-like |  |
| - pH | 14 (100%) |  |
| - Specific Gravity | 1.046 (Water = 1) |  |
| - Solubility | Easily soluble in cold water, hot water |  |

ENVIRONMENT & SAFETY:

- Hazard Rating 			Health –  3 	Fire –  0 	Reactivity – 0

- WHMIS				Class D-2B; Class E

STABILITY & REACTIVITY:

- The product is stable.

  - Highly reactive with acids.

  - Reactive with metals.

KEEP OUT OF REACH OF CHILDREN.

DANGER: Causes eye and skin burns. Causes severe respiratory tract irritation. Harmful if swallowed. May cause chronic toxic effects.

Precautions: Wear impervious gloves, chemical splash goggles and apron. Do not get in eyes, on skin or on clothing. Avoid breathing vapors, spray or mists. Wash thoroughly after handling.

FIRST AID

Eye contact: In case of contact, immediately flush eyes with cool running water. Remove contact lenses and continue flushing with plenty of water for at least 15 minutes. Get medical attention immediately. Skin contact: In case of contact, immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before reuse. Clean shoes thoroughly before reuse. Get medical attention immediately.

Inhalation: If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention immediately.

Ingestion: Rinse mouth; then drink one or two large glasses of water. Do not induce vomiting. Never give anything by mouth to an unconscious person. Get medical attention immediately.

FOR EMERGENCY MEDICAL INFORMATION IN USA OR CANADA, CALL 1-800-328-0026.

FOR EMERGENCY MEDICAL INFORMATION WORLDWIDE, CALL 1-651-222-5352 (USA).

READ MATERIAL SAFETY DATA SHEET BEFORE USING PRODUCT.

This document is available in French upon request

PACKAGING & PRODUCT NUMBERS:

| Size Available | Product Number |
| --- | --- |
| 2.5 U.S. Gallon/18.9 L | 14615 |

Ecolab Inc.

370 Wabasha Street, St. Paul, MN 55102 U.S.A.

Dist. By Ecolab Co. 5105 Tomken Road, Mississauga, Ont. Canada L4W2X5

www.ecolab.com 1-800-35-CLEAN