# Product Specification Document

Oasis Pro 30 Heavy Duty Alkaline Floor Cleaner

903731

CLASSIFICATION:

Liquid High Caustic Floor Cleaner

- SCOPE:

FOR INDUSTRIAL USE ONLY

A heavy duty floor cleaner for daily cleaning and degreasing of durable floor tile or other floors without finishes or waxes.

- DIRECTIONS FOR USE:

Do not mix with anything except for water.

- Open dispenser door, place bag in position and slide fitment into slot.

- Insert probe into bag, twist cap tight, and close dispenser.

- Wipe up spills and sweep floor.

- Fill mop bucket with solution.

- Apply solution with a mop.

- Leave on floor until solution penetrates soil.

- Remove stubborn stains with a stiff bristle brush.

- Mop up solution or squeegee towards floor drain. Rinse with fresh water and allow to air dry.

Mild Strength: 0.5 oz/gal (4 mL/L)

Medium Strength: 2 oz/gal (16 mL/L)

Heavy-Duty Strength: 6 oz/gal (48 mL/L)

For service or additional information, call 1-800-35-CLEAN (352-5326).

- PRODUCT INFORMATION:

State	Liquid

Appearance	Dark Green

Fragrance	Lemon/Ammonia Odor

pH – conc.	13.5	pH – use dilution – 10.6 – 12.5

% P 	0%

Conc. Test	Volumetric

Hazard Rating (HMIS)	Health - 3	Fire - 0	Reactivity - 0

- STABILITY & STORAGE:

Shelf life of concentrate:  2 Years

Usage Notes: